Homework

One-Page Rapid Test Design Summary: Virus X

# Objective

Design a lateral flow rapid antigen test for the hypothetical pathogen Virus X. The test should provide a visual result in approximately 10–20 minutes using a nasal-swab sample and should include both a Test line (T) and Control line (C).

# Target Antigen

Chosen target: Virus X surface nucleocapsid protein (VX-N). This viral protein is selected because it is assumed to be abundant in infected respiratory samples and to contain a unique, stable epitope that can be recognized specifically by antibodies. A virus-specific antigen is preferable to a host protein because its detection indicates the presence of Virus X material in the sample.

# Antibodies Required

| Antibody / reagent | Location | Role |
| --- | --- | --- |
| Anti-VX-N detector antibody, labeled with colored nanoparticles (for example, colloidal gold) | Conjugate pad | Binds VX-N antigen in the sample. Its colored nanoparticle label makes a visible line possible. |
| Second anti-VX-N capture antibody, recognizing a different VX-N epitope | Test line (T) | Captures the antigen–detector-antibody complex, producing a visible colored Test line when Virus X antigen is present. |
| Anti-species antibody, such as anti-mouse IgG | Control line (C) | Captures the labeled detector antibody whether or not antigen is present. A visible Control line verifies proper liquid flow and functional test reagents. |

# How the Test Works

A nasal-swab sample is mixed with extraction buffer and placed onto the sample pad.

Capillary action moves the liquid across the strip. If VX-N antigen is present, it binds the mobile, nanoparticle-labeled detector antibody in the conjugate pad.

At the Test line, the fixed anti-VX-N capture antibody binds the complex. The accumulated nanoparticles form a visible colored T line.

Excess labeled detector antibody continues to the Control line, where anti-species antibody captures it and forms a C line. The C line must appear for a valid test.

# Test Strip Sketch

Sample added here

↓

┌────────────┬─────────────────────┬────────────────────┬────────────────────┬──────────────┐

│ Sample pad │ Conjugate pad       │ Test line (T)      │ Control line (C)   │ Absorbent pad│

│ nasal swab │ labeled anti-VX-N   │ fixed anti-VX-N    │ anti-mouse IgG     │ draws fluid  │

│ + buffer   │ detector antibody   │ capture antibody   │ captures label     │ onward       │

└────────────┴─────────────────────┴────────────────────┴────────────────────┴──────────────┘

→ Capillary-flow direction →

Positive:    T line + C line

Negative:           C line only

Invalid:     no C line (with or without a T line)

# Result Interpretation

Positive result: Both Test and Control lines appear. This indicates that Virus X antigen was detected. Negative result: Only the Control line appears, meaning no detectable Virus X antigen was captured under the test conditions. Invalid result: The Control line is absent. The test must be repeated with a new device because sample migration or reagent function cannot be verified.

Note: In a real diagnostic product, the selected antigen and antibody pairs would require extensive validation for sensitivity, specificity, cross-reactivity, stability, and performance with clinical specimens before any clinical use.